# Proposed edits to Australian Drinking Water Guidelines Version 4.0– consequential edits related to revised ammonia, nickel, and chlorate fact sheets

Text in red are proposed edits to the existing text in the Australian Drinking Water Guidelines (the Guidelines).

| Edit no. | Guidelines section | Page no. | Current text | Suggested edit | Comments |
| --- | --- | --- | --- | --- | --- |
|  | Table 8.4 | 147 | Nickel NHMRC Health-Based Guideline Value (mg/L) – 0.02 | Nickel NHMRC Health-Based Guideline Value (mg/L) – 0.05 Revised text as per Table 8.4 below | Revise health-based guideline value and corresponding example doses by treatment chemical as per calculations in Table 8.4 |
|  | Table 9.5 Generic frequencies for monitoring non-microbial drinking water quality as supplied to the customer | 176 | See Table 9.5 below | Revised text as per Table 9.5 below | Inclusion of chlorate in table 9.5 |
|  | Table 10.6 Guideline values for physical and chemical characteristics | 210-225 | See Table 10.6 below | Update Table 10.6 with new health-based/aesthetic guidance values and comments for nickel, and chlorate | Updated for new health-based guideline values. |

## Table 8.4 Recommended maximum impurity concentrations for selected drinking water treatment chemicals*

|  |  |  | IMPURITY Nickel (mg/L) |
| --- | --- | --- | --- |
|  |  | NHMRC Health-Based Guideline Value (mg/L) | 0.02 0.05 |
| Treatment Chemical | Chemical Strength (%) | Example doses (mg/L) |  |
| Aluminium chlorohydrate | 23 | 100 (as Al2O3) | 4.6 11.5 |
| Aluminium sulfate (alum) | 47 | 20 (as Al2(SO4)3) | 47 117.5 |
| Aluminium sulfate (alum) | 47 | 60 (as Al2(SO4)3) | 15.7 39.2 |
| Aluminium sulfate (alum) | 47 | 120 (as Al2(SO4)3) | 7.8 19.6 |
| Calcium hydroxide | 99 | 30 (as Ca(OH)2) | 66 165.0 |
| Calcium hypochlorite | 65 | 3 (as Cl2) | 433.3 1,083.0 |
| Calcium oxide | 10 | 500 (as CaO) | 0.4 1.0 |
| Chlorine | 100 | 3 (as Cl2) |  |
| Copper sulfate | 25.5 | 1 (as CuSO4.5H2O) | 510 1,275.0 |
| Ferric chloride | 42 | 120 (as FeCl3) | 7 17.5 |
| Ferric sulfate | 20 | 100 (as Fe2(SO4)3) | 4 10.0 |
| Hydrochloric acid | 33 | 5 (as HCI) | 132 330.0 |
| Hydrofluorosilicic acid | 16 | 1.5 (as F) |  |
| Hydroxylated ferric sulfate | 12.5 | 100 | 2.5 6.3 |
| Polyaluminium chloride | 10 | 100 (as Al2O3) | 2.0 5.0 |
| Potassium permanganate | 99 | 1 (as KMnO4) |  |
| Sodium fluoride | 45 | 1.5 (as F) |  |
| Sodium fluorosilicate | 60 | 1.5 (as F) |  |
| Sodium hydroxide | 50 | 10 (as NaOH) | 100 250.0 |
| Sodium hypochlorite | 12 | 3 (as Cl2) | 80 200.0 |
| Sulfuric acid | 98 | 5 (as H2SO4) |  |

*NB. This excerpt from Table 8.4 illustrates the consequential updates if the nickel guideline value is increased from 0.02 mg/L to 0.05 mg/L.

## Table 9.5 Generic frequencies for monitoring non-microbial drinking water quality as supplied to the consumer

|  | Frequency of sampling - Weekly | Frequency of sampling - Monthly | Frequency of sampling - Quarterly | Frequency of sampling - Annually+ | Comments |
| --- | --- | --- | --- | --- | --- |
| Physical characteristics | pH Temperature Total dissolved solids¹ | Colour Turbidity Dissolved oxygen Hardness² |  | Taste and odour | ¹ If reverse osmosis used, or there are known salinity issues, otherwise quarterly ² If water is treated for hardness |
| Water treatment related chemicals (if used) | Fluoride¹ Aluminium² Chlorine Copper (seasonal) |  | Any related organic contaminants, e.g. acrylamide, carbon tetrachloride, epichlorohydrin |  |  |
| Disinfection byproducts (DBPs) |  | Trihalomethanes (THMs)¹𝄒² Ammonia, nitrite, nitrate² Bromate, formaldehyde³ Chlorite, chlorate⁴ |  |  | ¹ Where chlorine or chloramine are used. ² Where chloramine is used. ³ Where ozone is used. ⁴ Where chlorine dioxide or liquid chlorine hypochlorite is used. If detected at elevated concentrations, close to, or above guideline values, additional related DBPs should also be analysed. |
| Inorganics | Iron Manganese |  | Arsenic, nitrate, fluoride, selenium, lead, mercury¹ Ammonia, cadmium, chromium, nickel, zinc, copper, hydrogen sulfide | Tin, silver beryllium, uranium, iodide, molybdenum, boron, barium | ¹ Priority contaminants: quarterly sampling for groundwater sources, more frequent monitoring when detected at elevated concentrations; otherwise sampling reduced to annually, seasonally or event-related (e.g. storm events, reservoir turnover events). |
| Pesticides and organic toxicants |  | If detected or potential presence |  | If not detected | Monthly or quarterly sampling for pesticides/organic toxicants previously (or potentially) detected; seasonally annually, or event-related (e.g. storm events, spills) for other pesticides/organic toxicants. |
| Radiological |  |  |  | Radionuclides | New supplies should be assessed quarterly for one year, then every 2 years (groundwater) or 5 years (surface water). Increase frequency to quarterly if guideline screening levels exceeded. |
| Metals with potential for leaching |  |  |  | Bismuth, silicon, antimony, chromium, copper, nickel | Annual sampling, unless pipework material has been considered as part of the nominated sampling frequency. |

## Table 10.6 Guideline values for physical and chemical characteristics

| Characteristic | Guideline values (mg/L unless otherwise specified) |  | Comments |
| --- | --- | --- | --- |
|  | Health | Aesthetic |  |
| Ammonia (as NH3) | c | 0.5 | Presence may indicate sewage contamination and/or microbial activity. High levels may corrode copper pipes and fittings. Taste threshold for ammonium 35 mg/L. Odour threshold for ammonia 1.5 mg/L. |
| Chlorate | 0.8 c e |  | Occurs in water as a degradation product of hypochlorite solution and as a by-product of chlorine dioxide disinfection. Action to reduce chlorate is encouraged, but must not compromise disinfection, as non-disinfected water poses significantly greater risk than chlorate. By-product of chlorination. Insufficient data to set a health-based guideline value. |
| Chlorite | 0.8 |  | By-product of chlorine dioxide disinfection. Action to reduce chlorite is encouraged, but must not compromise disinfection, as non-disinfected water poses significantly greater risk than chlorite. |
| Nickel | 0.02 0.05 |  | Concentrations usually very low; but up to 0.5 mg/L reported after prolonged contact of water with nickel-plated fittings. |